SF-1P

Final Test Instructions

Last Updated: 28/12/2022

Contents

1	Required Test Equipment	3

2	Preparations for Testing	3

2.1	U-boot download	3

3	Visual Inspection	7

4	Basic Operational Tests (BOT)	8

4.1	Automated BOT	8

4.2	Manual BOT	8

5	Environmental Stress Screening (ESS)	17

6	Automated Final Tests	17

6.1	Setup	17

6.2	Running the Test	17

7	Manual Final Tests	17

7.1	General Function Tests	17

7.2	Identification test	17

7.3	FD Button check test	19

7.4	Dry Contact/Alarm test	21

7.5	Cellular Modem test (according to option)	23

7.6	TEST RS232 for only port s1 Option 2RS (according to option)	28

7.7	GPS test (according to option)	30

7.8	Wi-Fi test (according to option)	30

7.9	Data Test	36

7.10	Test SSH connection ETH port 4 or 6 test (according to option)	39

8	Preparations for Shipment	41

9	Factory Setting	42

10	Safety Test	42

Appendix A.	EEPROM Parameters	43

Appendix B.	ETX-204 generator configuration	45

# Required Test Equipment

| Item | Generic Name/Description | Manufacturer | Model |
| --- | --- | --- | --- |
|  | SF-1P - UUT | RAD |  |
|  | SFP for GE | RAD | SFP-9G |
|  | RJ45/DB9 Terminal and Serial cables | RAD | CBL-RJ45/D9/F/6FT x 2 |
|  | GETH Cable | N/A | N/A |
|  | Terminal S/W | N/A | Procomm |
|  | TFTP - S/W | N/A | N/A |
|  | GE/FE Generator | RAD | ETX-204AX x2 |
|  | Serial port LOOP cable | RAD | CBL-RS232-RJ45-RJ45 |
|  | GPS Passive antenna | RAD | T-RPA-PB0-393353-1M-X-005 |
|  | 32GB sd card | N/A | 32 GB capacity only! |

# Preparations for Testing

## U-boot download

- The process of loading u-boot software and UUT option (according to latest version in ECO) is performed by PC Linux only.

- Block diagram below (Figure 1) depicts connection of SF-1P to terminal

through serial port and connection of TFTP server to device Ethernet2

Figure 1 : SF-1P Terminal and Ethernet port connection

- Connect terminal to Serial Port 2 (upper connector) and relocate jumper J21 as shown in figure below:

- Transfer UART image to the SF-1P, set jumpers J18, J19, J20 should be installed for “UART Mode” and relocated as show in figure below:

- Set / check all parameters in the inventory (according to the circuit)

Pay attention that the “start from” area set to the first test

6.    Use following command:

cd /home/etx-1p/projects/boot/u-boot

sudo ./deploy.sh 2    Press "Enter" ( 2 is the serial number (USB port)

Waiting for the process to complete until the command:

PCPE>>

PCPE>>setenv serverip 10.10.10.1   (PC Linux NIC)

PCPE>>setenv ipaddr 10.10.10.10    (UUT)

PCPE>>setenv gatewayip 10.10.10.1

PCPE>>setenv ethact neta@40000

PCPE>>ping 10.10.10.1    (To check the communication to the server)

PCPE>>bubt flash-image.bin spi tftp

Waiting for the process to complete until the command:

Done!

PCPE>>tftpboot $loadaddr boot- scripts/set_boot_param_etx_general.img

Press "Enter"

Waiting for the process to complete until the command:

done

Bytes transferred = 3601 (e11 hex)

PCPE>>

PCPE>>source $loadaddr

Waiting for the process to complete until the command:

Erasing SPI flash...Writing to SPI flash...done

PCPE>>

PCPE>>reset

resetting ...

>

E

Press "Enter"

>x    Write the letter X    Press "Enter"

Sometimes you need to write the letter X again

To begin deploying the software

>x    Write the letter X

Need to see the command    Hit any key to stop autoboot:  0   and press “Enter"

PCPE>>

| NOTE: | Both files are burned according to an order option setenv fdt_name /boot/armada-3720-SF1p.dtb (option 2 LAN) setenv fdt_name /boot/armada-3720-SF1p_superSet.dtb (option 4 LAN ) |
| --- | --- |

PCPE>> setenv fdt_name /boot/armada-3720-SF1p.dtb

setenv fdt_name /boot/armada-3720-SF1p_superSet.dtb

| NOTE: | In PC, need 4x Mac per product |
| --- | --- |

PCPE>>setenv eth1addr 00:51:82:11:22:60

PCPE>>setenv eth2addr 00:51:82:11:22:61

PCPE>>setenv eth3addr 00:51:82:11:22:62

PCPE>>setenv eth4addr 00:51:82:11:22:63

PCPE>>saveenv

Waiting for the process to complete until the command:

Erasing SPI flash...Writing to SPI flash...done

PCPE>>

- For option 4 LAN

Enter command:

PCPE>>iic e 52

PCPE>>iic c  <by barcode name> .txt

Example:

iic c SF-1PE1DC4U2S2RS.txt

MODEM_1_MANUFACTURER=,MODEM_2_MANUFACTURER=,MODEM_1_TYPE=,MODEM_2_TYPE=,MAC_ADDRESS=18:18:18:18:18:AC,MAIN_CARD_HW_VERSION=0.4,SUB_CARD_1_HW_VERSION=,CSL=B,PART_NUMBER=SF-1P/E1/DC/4U2S/2RS,PCB_MAIN_ID=SF-1P.REV0.4I,PCB_SUB_CARD_1_ID=,PS=12V,SD_SLOT=YES,SERIAL_1=RS232,SERIAL_2=RS232,SERIAL_1_CTS_DTR=YES,SERIAL_2_CTS_DTR=YES,RS485_1=,RS485_2=,DRY_CONTACT=2_2,NNI_WAN_1=FIBER,NNI_WAN_2=FIBER,LAN_3_4=,LIST_REF=0.0,END=

- For option 2 LAN

Enter command:

PCPE>> iic c  <by barcode name> .txt

Example:

iic c SF-1PE1ACEX2U2RS.txt

MODEM_1_MANUFACTURER=,MODEM_2_MANUFACTURER=,MODEM_1_TYPE=,MODEM_2_TYPE=,MAC_ADDRESS=18:18:18:18:18:18,MAIN_CARD_HW_VERSION=0.4,SUB_CARD_1_HW_VERSION=,CSL=B,PART_NUMBER=SF-1P/E1/ACEX/2U/2RS,PCB_MAIN_ID=SF-1P.REV0.4I,PCB_SUB_CARD_1_ID=,PS=12V,SD_SLOT=YES,SERIAL_1=RS232,SERIAL_2=RS232,SERIAL_1_CTS_DTR=YES,SERIAL_2_CTS_DTR=YES,RS485_1=,RS485_2=,DRY_CONTACT=2_2,NNI_WAN_1=,NNI_WAN_2=,LAN_3_4=,LIST_REF=0.0,END=

PCPE>>

PCPE>>run bootnet

Waiting for the process to complete until the command:

[  373.381241] kvm: exiting hardware virtualization

[  373.404>

Press "Enter"

>

E

Press "Enter"

>x    Write the letter X    Press "Enter"

Sometimes you need to write the letter X again

To begin deploying the software

>x    Write the letter X    Press "Enter"

Waiting for the process to complete until the command:

user>su

password>1234

SF-1p#

- Power off SF-1P board.

- Change Jumpers as follow for normal mode:

- Power on SF-1P board - plug the power

- When the boot starts, hit any key to stop autoboot and get to the PCPE U-Boot prompt:

Need to see the command    Hit any key to stop autoboot:  0   and press "Enter"

PCPE>>

# Visual Inspection

- As specified in the GFTI.

- Confirm there is a Quectel cellular modem according to the ordering option L1, L2, L3, L4 or HSP.

- Make sure the mini - coax cables U.FL-to-SMA are attached to corresponding RF connectors on cellular modem:

- For LTE modems L1, L2, L3 and L4, LTE MAIN and LTE AUX are connected on the back panel.

- For HSP modem, LTE MAIN is connected on the back panel.

- For GPS option, GPS is connected on the back panel.

- Verify J18, J19, J20 assembled

# Basic Operational Tests (BOT)

## Automated BOT

N/A

## Manual BOT

All marked by yellow should be checked

### USB Tree Test (according to option)

- Power on SF-1P board - plug the power

- When the boot starts, hit any key to stop autoboot and get to the PCPE U-Boot prompt:

Need to see the command    Hit any key to stop autoboot:  0  and press "Enter"

PCPE>> usb start

starting USB...

USB0:   Register 2000104 NbrPorts 2

Starting the controller

USB XHCI 1.00

USB1:   USB EHCI 1.00

scanning bus 0 for devices... 2 USB Device(s) found

scanning bus 1 for devices... 1 USB Device(s) found

scanning usb for storage devices... 0 Storage Device(s) found

| option | bus 0 devices |
| --- | --- |
| LTE and WiFi | 2 |
| LTE only | 2 |
| WiFi only | 1 |
| No LTE no WiFi | 1 |

PCPE>> usb tree

USB device tree:

1 Hub (5 Gb/s, 0mA)

| U-Boot XHCI Host Controller

|

+-2 Vendor specific

1 Hub (480 Mb/s, 0mA)

u-boot EHCI Host Controller

| option | 2 Vendor specific |
| --- | --- |
| LTE and WiFi | 2 Vendor specific |
| LTE only (except L450x) | 2 Vendor specific |
| WiFi only | no “2 Vendor specific” |
| No LTE no WiFi | no “2 Vendor specific” |
| LTE L450x only | no “2 Vendor specific” |

PCPE>> pci

Scanning PCI devices on bus 0

BusDevFun  VendorId   DeviceId   Device Class       Sub-Class

_____________________________________________________________

00.00.00   0x1b4b     0x2b42     Network controller      0x00

### SD-Card connectivity test

PCPE>> mmc info

Ignore the first print

PCPE>> mmc info

Device: sdhci@d0000

Manufacturer ID: 3

OEM: 5344

Name: SC32G

Tran Speed: 50000000

Rd Block Len: 512

SD version 3.0

High Capacity: Yes

Capacity: 29.7 GiB

Bus Width: 4-bit

Erase Group Size: 512 Bytes

### SOC Flash memory device test

- Power on SF-1P board - plug the power

- When the boot starts, hit any key to stop autoboot and get to the PCPE U-Boot prompt:

Need to see the command    Hit any key to stop autoboot:  0   and press “Enter"

PCPE>>

- Use following command:

PCPE>> mmc dev 1:0

switch to partitions #0, OK

mmc1(part 0) is current device

Device: sdhci@d8000

Manufacturer ID: 90

OEM: 14a

Name: H8G4a

Tran Speed: 52000000

Rd Block Len: 512

MMC version 5.1

High Capacity: Yes

Capacity: 7.3 GiB

Bus Width: 8-bit

Erase Group Size: 512 KiB

HC WP Group Size: 8 MiB

User Capacity: 7.3 GiB WRREL

Boot Capacity: 8 MiB ENH

RPMB Capacity: 4 MiB ENH

PCPE>> mmc list

sdhci@d0000: 0 (SD)

sdhci@d8000: 1 (eMMC)

### SOC I2C test

- Power on SF-1P board - plug the power

- When the boot starts, hit any key to stop autoboot and get to the PCPE U-Boot prompt:

Need to see the command    Hit any key to stop autoboot:  0   and press “Enter"

PCPE>>

- Use following command:

PCPE>> i2c bus

Bus 0:  i2c@11000

PCPE>> i2c dev 0

Setting bus to 0

PCPE>> i2c probe

Valid chip addresses: 20 21 22 23 24 25 26 27 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F 40 41 42 43 44 45 46 47 48 49 4A 4B 4C 4D 4E 4F 50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F 60 61 62 63 64 65 66 67 68 69 6A 6B 6C 6D 6E 6F 70 71 72 73 74 75 76 77 78 79 7A 7B 7C 7D 7E 7F

PCPE>> i2c mw 0x52 0.2 0xaa 0x1

PCPE>> i2c md 0x52 0.2 0x20

0000: aa

PCPE>> i2c mw 0x52 0.2 0xbb 0x1

PCPE>> i2c md 0x52 0.2 0x20

0000: bb

### Front panel LEDs test

SF-1P Front Panel LEDs

Figure 2: SF-1P Front Panel LEDs

- Connect a power supply to a test product SF-1P and confirm a green LED PWR is on

- Disconnect the power supply to the test product SF-1P and confirm the LED PWR is off

LEDs may be tested with PCPE commands:

- Power on SF-1P board - plug the power

- When the boot starts, hit any key to stop autoboot and get to the PCPE U-Boot prompt:

Need to see the command    Hit any key to stop autoboot:  0   and press “Enter"

PCPE>>

| Test LED | Command | LED Status |
| --- | --- | --- |
| RUN” on/off | gpio toggle GPIO112 gpio toggle GPIO112 | GREEN OFF |
| ALM on/off | gpio toggle GPIO113 gpio toggle GPIO113 | RED OFF |
| AUX Green LED on | mii write 1 1 0x80fe mii write 1 0 0x9656 | GREEN |
| AUX Green LED off | mii write 1 1 0x80ee mii write 1 0 0x9656 | OFF |
| AUX Red LED on | mii write 1 1 0x80fe mii write 1 0 0x9636 | RED |
| AUX Red LED off | mii write 1 1 0x80ee mii write 1 0 0x9636 | OFF |
| WIFI Led on | mii write 1 1 0x80fe mii write 1 0 0x9676 | GREEN |
| WIFI Led off | mii write 1 1 0x80ee mii write 1 0 0x9676 | OFF |
| Led SIM1 on | mii write 1 1 0x90fe mii write 1 0 0x9656 | GREEN |
| Led SIM1 off | mii write 1 1 0x90ee mii write 1 0 0x9656 | OFF |
| Led SIM2 on | mii write 1 1 0x90fe mii write 1 0 0x9636 | GREEN |
| Led SIM2 off | mii write 1 1 0x90ee mii write 1 0 0x9636 | OFF |
| Sfp port 1 on | mii write 2 1 0x80ef mii write 2 0 0x96B6 | GREEN |
| Sfp port 1 off | mii write 2 1 0x80ee mii write 2 0 0x96B6 | OFF |
| Sfp port 2 on | mii write 2 1 0x80fe mii write 2 0 0x96B6 | GREEN |
| Sfp port 2 off | mii write 2 1 0x80ee mii write 2 0 0x96B6 | OFF |
| UTP Port 3 on | mii write 1 1 0x80ef mii write 1 0 0x9636 | GREEN |
| UTP Port 3 off | mii write 1 1 0x80ee mii write 1 0 0x9636 | OFF |
| UTP Port 4 on | mii write 1 1 0x80ef mii write 1 0 0x9656 | GREEN |
| UTP Port 4 off | mii write 1 1 0x80ee mii write 1 0 0x9656 | OFF |
| UTP Port 5 on | mii write 1 1 0x80ef mii write 1 0 0x9676 | GREEN |
| UTP Port 5 off | mii write 1 1 0x80ee mii write 1 0 0x9676 | OFF |
| UTP Port 6 on | mii write 1 1 0x80ef mii write 1 0 0x9696 | GREEN |
| UTP Port 6 off | mii write 1 1 0x80ee mii write 1 0 0x9696 | OFF |
| Ser1 TX on | mii write 1 1 0x90ef mii write 1 0 0x9636 | GREEN |
| Ser1 TX off | mii write 1 1 0x90ee mii write 1 0 0x9636 | OFF |
| Ser1 RX on | mii write 1 1 0x90ef mii write 1 0 0x9676 | GREEN |
| Ser1 RX off | mii write 1 1 0x90ee mii write 1 0 0x9676 | OFF |
| Ser2 TX on | mii write 1 1 0x90ef mii write 1 0 0x9656 | GREEN |
| Ser2 TX off | mii write 1 1 0x90ee mii write 1 0 0x9656 | OFF |
| Se2 RX on | mii write 1 1 0x90ef mii write 1 0 0x9696 | GREEN |
| Se2 RX off |  | OFF |

Table 1: LED TESET IS ALL commands

### Front panel LED LTE test

Figure 3: SF-Pront Panel LED LTE

- The LEDs may be tested with LINUX commands:

- Go into SF-1P  Linux shell the following:

user>su

password>1234

SF-1p# logon debug

Key code:                    8388569129   ( Example)

Challenge code: (10 figure numbers)

Open Deccryptor

- Enter the code   (Challenge code:  in the field key)

- Push on (change pass), you will get a number (Result)

- Insert the number to UUT

password>**********

SF-1p# debug shell

[root@localhost /]#

see the command:

[root@localhost /]# stty icrnl

[root@localhost /]# cat > lte_ledtest.sh

Open file download with notepad

ctrl+D

[root@localhost /]# cat > lte_ledbar_test.sh

Open file download with notepad

ctrl+D

[root@localhost /]# chmod 777 lte_ledtest.sh

[root@localhost /]# chmod 777 lte_ledbar_test.sh

[root@localhost /]# ./lte_ledbar_test.sh

- Verify that you see that all 4 LEDs are lit in green

- Verify that all 4 LEDs are off

# Environmental Stress Screening (ESS)

1.	As specified in the GFTI.

# Automated Final Tests

The following instructions reflect:

|  | Last Updated |
| --- | --- |
| Final Test Instructions | 28.12.2022 |
| Automatic Tester | 28.12.2022 |

## Setup

1.	Connect the Tester’s cables to the following UUT’s ports: PWR, 2xSFP, 4xUTP, S1 and S2 according to order option.

2.	According to order option insert two SIM cards.

3.	According to order option connect Cellular, GPS and Wi-Fi antennas.

4.	Scan the DUT’s ID barcode to configure the Tester accordingly.

## Running the Test

1.	Press the “RUN” button to start running the test and follow the Instructions on the screen. If the test fails, the display will show a red Fail message and the test will be stopped, and you may resume testing from this point.

2.	Verify that the test passed.

# Manual Final Tests

## General Function Tests

Note for all PCB 0.4 need to power on/off for 5 time and check unit is response

Need to verify voltage is up then 3.25 on TP2

## Identification test

- Type as follows:

user>su

Password:1234

- Verify the ID of Chassis and UUT’s are according to ECO and EEPROM following commands:

config  system

config>system# show device-information

| ID |  |
| --- | --- |
| Description | SW: Verify the Versions list (According to the latest ECO by order option) HW: verify (According to the latest ECO) and PCB according to Option |
| MAC Address | Verify range: 18:06:F5:00:00:00 - 18:06:F5:FF:FF:FF or 00:20:D2: 00: 00: 00 - 00:20:D2: FF: FF: FF |
| Model | according to an order option SF-1P (option 2 LAN) or SF-1P superset (option 4 LAN ) |
| Name | SF-1p |

Example: Bold in yellow

SF-1p>config>system# show summary-inventory

| FW Ver | according to Option (ORDER) |
| --- | --- |

- Connect Ethernet Generator to all WAN and LAN port and verify the ports SFP, RJ45 (According to by order option)

following commands:

exit all

configure port ethernet 1

config>port>eth(1)#no shutdown

config>port>eth(1)#show status

Note: Please execute the same commands to the following ports ethernet 2-6

| ethernet 1 - 2 | Connector Type | SFP In |
| --- | --- | --- |
|  | Administrative Status | up |
|  | Operational Status | up |
| Identification SFP port ethernet 1 - 2 | Manufacturer Part Number | SFP-9G |
| ethernet 3-6 | Connector Type | RJ45 |
|  | Administrative Status | up |
|  | Operational Status | up |

- Power off and on SF-1P board - plug the power

- When the boot starts, hit any key to stop autoboot and get to the PCPE U-Boot prompt:

Need to see the command    Hit any key to stop autoboot:  0   and press “Enter"

PCPE>>

- Verify the UBOOT (According to the latest ECO by order option)

U-Boot 2017.03.VER1.0.2-armada-17.10.2 (Nov 22 2021)

- Verify the DRAM:  1 GiB  (According to order option)

Example:U-BOOT,DRAM

PCPE>> printenv NFS_VARIANT

- Verify NFS_VARIANT=general (According to the latest ECO by order option)

## FD Button check test

Figure 4 : SF-1P FD BUTTON (factory-default)

- The test for factory-default-config

Use following command:

user>su

password>1234

SF-1p#

- Press the button for 15 seconds as shown in Figure 4

- Verify that the device is booting. Verify the message factory-default-config

## Dry Contact/Alarm test

- Prepare the product according to the drawing

Connect the connector to Dry Contact as shown

- The Dry Contact may be tested with LINUX commands:

SF-1p# logon debug

Key code:                    8388569129   ( Example)

Challenge code: (10 figure numbers)

Open Deccryptor

- Enter the code   (Challenge code:  in the field key)

- Push on (change pass), you will get a number (Result)

- Insert the number to UUT

password>**********

sf-1p# debug shell

[root@localhost /]#

see the command:

Open file with notepad and download the script to the UUT

Change the 2 switches upwards according to the picture and check 2 LEDs on

Change the 2 switches downwards according to the picture and check 2 LEDs off

## Cellular Modem test (according to option)

7.5.1 Test for one Cellular Modem

- Connect cellular Antenna to LTE MAIN.

- Test Cellular SIM1,SIM2, the commands:

user>su

Password:1234

configure

port

cellular lte

sim 1

apn-name "statreal"

exit

mode sim 1

no shutdown

exit

exit

router 1

interface 1

bind cellular lte

dhcp

dhcp-client

client-id mac

exit

no shutdown

exit all

- check the parameters the modem  sim 1

configure port cellular lte

config>port>cellular(lte)# show status

| Cellular network connection | Connected |
| --- | --- |
| IMEI | 866758044466866 (Example) |
| RSSI (dBm) | -51 : -75 |
| Administrative Status | Up |
| Operational Status | Up |
| Mode | SIM 1 |
| SIM Status | ready |
| Firmware | Check firmware version |

Example: firmware version

| Model | Latest FW |
| --- | --- |
| EC25-AUF (L3) | EC25AUFAR06A01M4G |
| UC20G (HSP) | UC20GQDR03A17E1G |
| EC-25EFA (L1) | EC25EFAR06A03M4G |
| EC-25AFA (L2) | EC25AFAR05A04M4G |
| EC25AFFD (L4) | EC25AFFDR07A09M4G |

Example:

Ping 8.8.8.8

Example ping :

- Check the no packet loss

Switch to sim2:

exit all

configure port cellular lte

config>port>cellular(lte)# shutdown

sim 2

apn-name "statreal"

exit

mode sim 2

no shutdown

exit all

configure port cellular lte

config>port>cellular(lte)# no shutdown

- check the parameters the modem  sim 2

config>port>cellular(lte)# show status

| Cellular network connection | Connected |
| --- | --- |
| RSSI (dBm) | -51 : -75 |
| Administrative Status | Up |
| Operational Status | Up |
| Mode | SIM 2 |
| SIM Status | ready |

Example:

Ping 8.8.8.8

Example ping :

- Check the no packet loss

### L4 Cellular Modem

1.Connect cellular Antenna to LTE MAIN.

2.Test Cellular SIM1, the commands:

user>su

Password:1234

Configure

port

cellular lte

mode sim 1

sim 1

apn-name "statreal"

pdp-type relayed-ppp

exit

no shutdown

exit all

configure router 1

interface 1

bind cellular lte

no shutdown

exit all

- check the parameters the modem  sim 1

- configure port cellular lte

- config>port>cellular lte > show status

| Cellular network connection | Connected |
| --- | --- |
| IMEI | 866758044466866 (Example) |
| RSSI (dBm) | -51 : -75 |
| Administrative Status | Up |
| Operational Status | Up |
| Mode | Sim1 |
| SIM Status | ready |
| Firmware | Check firmware version |

| Model | Latest FW |
| --- | --- |
| EC25AFFD (L4) | EC25AFFDR07A09M4G |

Ping 8.8.8.8

- Check the no packet loss

Switch to sim2:

exit all

configure port cellular lte

config>port>cellular lte > shutdown

sim 2

apn-name "statreal"

exit

mode sim 2

no shutdown

exit all

configure port cellular lte

config>port>cellular(lte)# no shutdown

- check the parameters the modem  sim 2

config>port>cellular(lte)# show status

| Cellular network connection | Connected |
| --- | --- |
| RSSI (dBm) | -51 : -75 |
| Administrative Status | Up |
| Operational Status | Up |
| Mode | SIM 2 |
| SIM Status | ready |

:

| Model | Latest FW |
| --- | --- |
| EC25AFFD (L4) | EC25AFFDR07A09M4G |

Ping 8.8.8.8

Example ping :

- Check the no packet loss

### dual modem

Cellular Modem test (according to option)

1.	Connect cellular Antenna to LTE MAIN.

2.	Test Cellular SIM1,SIM2, the commands:

user>su

Password:1234

configure

port

cellular lte-1

sim 1

apn-name "statreal"

exit

no shutdown

exit

cellular lte-2

sim 1

apn-name "statreal"

exit

no shutdown

exit

exit

router 1

interface 1

bind cellular lte-1

dhcp

dhcp-client

client-id mac

exit

no shutdown

exit

interface 2

bind cellular lte-2

dhcp

dhcp-client

client-id mac

exit

no shutdown

exit

exit all

•	check the parameters the modem  sim 1

configure port cellular lte-1

config

port

cellular lte 1

show status

Cellular network connection	Connected

IMEI	866758044466866 (Example)

RSSI (dBm)	-51:-75

Administrative Status	Up

Operational Status	Up

Mode	SIM 1

SIM Status   	ready

Firmware: Check firmware version according to the table below:

| Model | Latest FW |
| --- | --- |
| EC25-AUF (L3) | EC25AUFAR06A01M4G |
| UC20G (HSP) | UC20GQDR03A17E1G |
| EC-25EFA (L1) | EC25EFAR06A03M4G |
| EC-25AFA (L2) | EC25AFAR05A04M4G |

Example:

Ping8.8.8.8

Check no ping lost

Example:

Exit all

configure port cellular lte-2

config

port

cellular lte 2

show status

Cellular network connection	Connected

IMEI	866758044466866 (Example)

RSSI (dBm)	-51:-75

Administrative Status	Up

Operational Status	Up

Mode	SIM 1

SIM Status   	ready

Firmware	Check firmware version according to the table below:

| Model | Latest FW |
| --- | --- |
| EC25-AUF (L3) | EC25AUFAR06A01M4G |
| UC20G (HSP) | UC20GQDR03A17E1G |
| EC-25EFA (L1) | EC25EFAR06A03M4G |
| EC-25AFA (L2) | EC25AFAR05A04M4G |

Example:

Ping8.8.8.8

Check no ping lost

Example:

exit all

logon debug

copy the key code to Decryptor

press on “change pass”

copy the result and past them to password line in procomm

debug shell

ping 8.8.8.8 -I wwan0 -c 4

check 8.8.8.8 ping statistics for 0% packet loss

example:

[root@localhost /]# ping 8.8.8.8 -I wwan0 -c 4

PING 8.8.8.8 (8.8.8.8) from 46.210.111.173 wwan0: 56(84) bytes of data.

64 bytes from 8.8.8.8: icmp_seq=1 ttl=117 time=153 ms

64 bytes from 8.8.8.8: icmp_seq=2 ttl=117 time=37.3 ms

64 bytes from 8.8.8.8: icmp_seq=3 ttl=117 time=37.4 ms

64 bytes from 8.8.8.8: icmp_seq=4 ttl=117 time=44.7 ms

--- 8.8.8.8 ping statistics ---

4 packets transmitted, 4 received, 0% packet loss, time 3001ms

rtt min/avg/max/mdev = 37.350/68.154/153.077/49.123 ms

## TEST RS232 for only port s1 Option 2RS (according to option)

Note: Only If have two ports S1,S2

- Set the bellow setup:

Figure 5 : SF-1P Test for RS232 the port s1 only

•	The RS232 may be tested with LINUX commands:

•	Go into SF-1P  Linux shell the following:

user>su

password>1234

SF-1p# logon debug

Key code:                    8388569129   ( Example)

Challenge code: (10 figure numbers)

Open Decryptor

•	Enter the code   (Challenge code:  in the field key)

•	Push on (change pass), you will get a number (Result)

•	Insert the number to UUT

password>**********

SF-1p# debug shell

[root@localhost /]#

see the command:

Please connect a terminal cable to port s1 ,s2  according to the drawing

echo "1" > /sys/class/gpio/gpio484/value

stty -F /dev/ttyMV1 115200

Send data to Serial Port s2  terminal through port s1

echo "Data to terminal of Serial Port S1 from port S2 terminal" > /dev/ttyMV1

•	Confirm that message appears on terminal of Port s1

In s2 terminal enter command (receive data from Serial Port 1):

cat /dev/ttyMV1 &

-	Type any message ABCD into terminal of Serial Port S1 and press “Enter”

-	Confirm that message appears on port S2 terminal window.

| NOTE: | RS485 Product not supported |
| --- | --- |

## GPS test (according to option)

the commands:

config>system>clock>gnss(1)# secondary-system glonass galileo beidou

config>system>clock>gnss(1)# no shutdown

config>system>clock>gnss(1)# show status

- check the parameters

| Administrative Status : Up | up |
| --- | --- |
| Operational Status | up |
| Tracking Status | GNSS Locked |

## Wi-Fi test (according to option)

Note: Place the computer with WiFi adaptor in a distance from UUT

- Set the bellow setup:

Figure 6 : SF-1P Test WI-FI

### 2.4G Baud rate test

- Connect antenna for testing   WIFI MAIN

- Configure WiFi module to predefined channel (free) in 2.4G band and transmit over  WIFI MAIN antenna with optimal power

- Perform the bellow commands:

user>su

password>1234

Configure system dhcp-server 1

pool "1"

network 50.50.50.0/24

address-range 50.50.50.50 50.50.50.52

exit

no shutdown

exit

exit

port

wlan 1

radio-mode 802.11g

channel 4

access-point 1

ssid "2G"

password "1234" hash

max-clients 8

no shutdown

exit all

configure

router 1

interface 1

address 50.50.50.1/24

bind wlan 1 access-point 1

dhcp-client

client-id mac

exit

no shutdown

exit all

- Check the following parameters:

- Click an icon on your computer in the red bar on the image

Example:

- Connect with the following

SSID: 2G

- Click on  properties and verify the details:

| SSID | 2G |
| --- | --- |
| PROTOCOL | 802.11g (only 2.4G) |
| IPv4 address | 50.50.50.52 Example |

Example:

- Test communication from SF-1P to  PC over  the established WLAN:

Send ping 50.50.50.52 (Example)  and verify reply with no loss

- Open RUN command screen on PC

Check the signal percentage with the bellow commands:

Example:

- Disconnect Antenna  WIFI MAIN and check the parameters that percentage of   signal will decrease

Example:

### 5G Baud rate test

- Reset device

- Connect Antenna for testing to  WIFI AUX

- Configure WiFi module to predefined channel (free) in 5G band and transmit over  WIFI AUX antenna with optimal power

- Perform the bellow commands:

configure system dhcp-server 1

pool "1"

network 50.50.50.0/24

address-range 50.50.50.50 50.50.50.52

exit

no shutdown

exit

exit

port

wlan 2

radio-mode 802.11na

channel 36

access-point 1

ssid "5G"

password "1234" hash

max-clients 8

no shutdown

exit all

configure

router 1

interface 1

address 50.50.50.1/24

bind wlan 2 access-point 1

dhcp-client

client-id mac

exit

no shutdown

exit all

- Check the following parameters:

Click an icon on your computer in the red bar on the image

Example:

- Connect with the following the ssid 5G

- Click on  properties and verify the details:

Example:

- Check the following parameters:

| SSID | 5G |
| --- | --- |
| PROTOCOL | 802.11a (only 5G) |
| IPv4 address | 50.50.50.50 Example |

- Test communication from SF-1P to  PC over  the established WLAN:

- Send ping 50.50.50.50 (Example) and verify reply with no loss

- Open RUN command screen on PC

Check the signal percentage with the bellow commands:

Example:

## Data Test

Figure 7: SF-1P Test DATA running 6 ports

- Testing for 6 ports (according to option)

Note: port ethernet 1-2 connect with SFP-9G  for according to option order only SFP

- Type as follows:

- The Cellular is tested with LINUX commands:

ETX-1p# logon debug

Key code:                    8388569129   ( Example)

Challenge code: (10 figure numbers)

Open Decryptor

- Enter the code (Challenge code:  in the field key)

- Push on Change Pass, you will get a number (Result)

- Insert the number to UUT

password>**********

ETX-1p# debug shell

[root@localhost /]#

brctl addbr br0;

brctl addif br0 wan1;

brctl addif br0 wan2;

ifconfig wan1 up;

ifconfig wan2 up;

ifconfig br0 up;

brctl addbr br1;

brctl addif br1 lan0;

brctl addif br1 lan1;

ifconfig lan0 up;

ifconfig lan1 up;

ifconfig br1 up;

brctl addbr br2;

brctl addif br2 lan3;

brctl addif br2 lan2;

ifconfig lan2 up;

ifconfig lan3 up;

ifconfig br2 up;

- Configure the ETX-204A Generator 1 and 2 according to ‎Appendix B

- Verify DATA running without errors for 1 minutes in the ETX-204 generator

- confirm each generator has a Link-State is Up, “PRBS_OK” with running numbers and ERR-CNT = 0, PRBS_ERR=0, FRAME_ERR=0, SEQ_ERR=0.
 follows:

Example:

- Testing for 2 ports (according to option)

Figure 7 : SF-1P Test DATA running 2 ports

- Type as follows:

brctl addbr br1;

brctl addif br1 lan0;

brctl addif br1 lan1;

ifconfig lan0 up;

ifconfig lan1 up;

ifconfig br1 up;

- Configure the ETX-204A Generator 1 only  according to ‎Appendix B

- Verify DATA running without errors for 1 minutes in the ETX-204 generator

- confirm each generator has a Link-State is Up, “PRBS_OK” with running numbers and ERR-CNT = 0, PRBS_ERR=0, FRAME_ERR=0, SEQ_ERR=0.
 follows:

Example:

## Test SSH connection ETH port 4 or 6 test (according to option)

At the end of all the tests we find the device with the default IP 169.254.1.1/16

Connect terminal to Serial Port 2 (upper connector) and relocate jumper J21 as shown in figure below:

Figure 8 : SF-1P Test SSH

- Open SSH session with SF-1P device Ethernet Port4 or port 6 for Order option.

- Open Putty for ssh session and connect to SF-1P device (see below). Perform authorization into opened ssh session window:

- Click the open button

- Type as follows:

user>su

Password:1234

SF-1p#

# Preparations for Shipment

- As specified in the GFTI.

- SFP ports covers.

- Verify correct voltage indications

- Make sure the screw-nut of the coax connectors (ANT & MI/DV) is closed.

- Confirm the existence of the bar code label.

# Factory Setting

- As specified in the GFTI.

- Return to factory setting:

- admin factory-default

- Current configuration will be erased and device will reboot with factory default configuration. Are you sure? [yes/no]_yes

# Safety Test

As specified in the GFTI.

EEPROM Parameters

EEPROM parameters (By order option)

Example of EEPROM identifying string

MODEM_1_MANUFACTURER=QUECTEL,MODEM_2_MANUFACTURER=QUECTEL,MODEM_1_TYPE=EC25­E,MODEM_2_TYPE=EC25­E,MAC_ADDRESS=00:a7:b1:46:4e:a3,MAIN_CARD_HW_VERSION=0.0,SUB_CARD_1_HW_VERSION=0.1,CSL=A,PART_NUMBER=ETX­1P/ACEX/1SFP1UTP/4UTP,PCB_MAIN_ID=SF­1V/MAIN/0.1I,PCB_SUB_CARD_1_ID=SF­1V/PS/0.1I,PS=WDC­12V,SD_SLOT=YES,SERIAL_1=RS232,SERIAL_2=RS232,SERIAL_1_CTS_DTR=YES,SERIAL_2_CTS_DTR=YES,RS485_1=,RS485_2=,DRY_CONTACT=YES,NNI_PORT_4=COPPER,NNI_PORT_5=COPPER,PORT_2_3=YES,LIST_REF=0.0,END=

| Name | Possible Values | Comments |
| --- | --- | --- |
| MODEM_1_MANUFACTURER | QUECTEL SIERRA WIRELESS RAK | Null (no value) if modem is not present LoRa ordering option |
| MODEM_2_MANUFACTURER | QUECTEL AZUREWAVE RAK | Null (no value) if modem is not present WiFi, WF ordering option LoRa ordering option |
| MODEM_1_TYPE | UC20 EC25-E EC25-A EC25-AU EC25-AFFD EM9191 9XX 8XX LRC | Null (no value) if modem is not present QUECTEL, HSP ordering option QUECTEL, L1 ordering option QUECTEL, L2 ordering option QUECTEL, L3 ordering option QUECTEL, L4 ordering option SIERRA WIRELESS EM9191 5G RAK, LoRa ordering option LRA RAK, LoRa ordering option LRB RAK, LoRa ordering option LRC |
| MODEM_2_TYPE | UC20 EC25-E EC25-A EC25-AU EC25-AFFD AW-CM276MA 9XX 8XX LRC | Null (no value) if modem is not present QUECTEL, HSP ordering option QUECTEL, L1 ordering option QUECTEL, L2 ordering option QUECTEL, L3 ordering option QUECTEL, L4 ordering option AZUREWAVE, WF ordering option RAK, LoRa ordering option LRA (LoRa gateway) RAK, LoRa ordering option LRB (LoRa gateway) RAK, LoRa ordering option LRC (LoRa client) |
| MAC_ADDRESS | AA:BB:CC:DD:EE:FF | 10 consecutive addresses, starting with the specified |
| MAIN_CARD_HW_VERSION |  | Filled by Badas |
| SUB_CARD_1_HW_VERSION |  | Null (no value) if there is no sub card |
| HARDWARE_ADDITION | A B | Compatibility with PCB 5.0 and lower (PCB 6.0) PCB 6.0 with new features BOM |
| CSL |  | Filled by Badas |
| PART_NUMBER |  | Refer to [1P Ordering Options] for possible values |
| PCB_MAIN_ID |  | Filled by Badas |
| PCB_SUB_CARD_1_ID |  | Null (no value) if there is no sub card |
| PS | 12V DC WDC-I 12V-I | 12V DC 10V-30V non isolated (marketing DC/ACEX) Wide range 10V-60V non isolated (marketing DC) Wide range 20V-60V DC isolated (marketing WDC) 12V isolated (marketing 12V) |
| SD_SLOT | YES | Null (no value) if SD card slot is not present |
| SERIAL_1 | RS232 RS485 | Null (no value) if serial is not present Non isolated RS232 Non isolated RS485 |
| SERIAL_2 | RS232 RS485 | Null (no value) if serial is not present Non isolated RS232 Non isolated RS485 |
| SERIAL_1_CTS_DTR | YES | Null (no value) if serial is not present CTS and DTR are supported |
| SERIAL_2_CTS_DTR | YES | Null (no value) if serial is not present CTS and DTR are supported |
| RS485_1 | 2W | Null (no value) if RS485 is not supported Half duplex RS485 |
| RS485_2 | 2W | Null (no value) if RS485 is not supported Half duplex RS485 |
| DRY_CONTACT_IN_OUT | 2_2 3_1 | Null (no value) if there are no dry contacts 2 in, 2 out dry contacts 3 in, 1 out dry contacts |
| NNI_WAN_1 | FIBER | Null (no value) if there is no interface NNI port 1 is SFP |
| NNI_WAN_2 | COPPER FIBER | Null (no value) if there is no interface NNI port 2 is RJ-45 NNI port 2 is SFP |
| LAN_3_4 | YES | Null (no value) if ports are not present (baby product) MNI ports 3-4 are available |
| LIST_REF | 0.0 | Version of this EEPROM string |
| END |  | Null (no value), denoting end of string |

ETX-204 generator configuration

- Configure the parameters as shown below in yellow

Main Menu

3. Generator                          >

>3   Press "Enter"

ETX-204A-AC

*Generator

1. Generator number              >   (All)

2. Generator mode                    (GE)

3. Packet type                   >   (MAC)

8. Packet rate[1 - 1500000]      ... (115000)

14. Save

>14  Press "Enter"

ETX-204A-AC

*Generator

1. Generator number              >   (All)

>1   Press "Enter"

ETX-204A-AC

Generator>Generator number (Generator 1)

1. Generator 1

2. Generator 2

3. Generator 3

4. Generator 4

5. All

>1

ETX-204A-AC

*Generator

1. Generator number              >   (Generator 1)

14. Save

>14  Press "Enter"

ETX-204A-AC

Generator

1. Generator number              >   (Generator 1)

11. Base SA/DA                    >

>11    Press "Enter"

ETX-204A-AC

Generator>Base SA/DA

1. Generator num  >   (Generator 1-2)

| Generator | Base DA | Base SA |
| --- | --- | --- |
| Generator1 (WAN1) | 00-00-00-00-00-05 | 00-00-00-00-00-01 |
| Generator2 (WAN2) | 00-00-00-00-00-01 | 00-00-00-00-00-05 |

>S

Please select item <1 to 13>

S-Save to FPGA

ETX-204A-AC

*Generator

9. Generator statistics          []

>9  Press "Enter"

ETX-204A-AC

Generator>Generator statistics

>

H-Stop; C-Clear all; F-Clear one; S-Save to FPGA

- Configure the ETX-204A Generator 2 as follows:

- Configure the parameters as shown on the page that appears in yellow

- At the end of the process make sure all the parameters are correct

ETX-204A-AC

*Generator

1. Generator number              >   (All)

2. Generator mode                    (GE)

3. Packet type                   >   (MAC)

8. Packet rate[1 - 1500000]      ... (125000)

14. Save

>14 Press "Enter"

ETX-204A-AC

Generator

1. Generator number              >   (All)

>1 Press "Enter"

ETX-204A-AC

Generator>Generator number (All)

1. Generator 1

2. Generator 2

3. Generator 3

4. Generator 4

5. All

>1 Press "Enter"

ETX-204A-AC

Generator>Base SA/DA

1. Generator num  >   (Generator 1-4 )

| Generator | Base DA | Base SA |
| --- | --- | --- |
| Generator1 (LAN1) | 00-00-00-00-00-10 | 00-00-00-00-00-11 |
| Generator2 (LAN2) | 00-00-00-00-00-11 | 00-00-00-00-00-10 |
| Generator3 (LAN3) | 00-00-00-00-00-07 | 00-00-00-00-00-03 |
| Generator4 (LAN4) | 00-00-00-00-00-03 | 00-00-00-00-00-07 |

>S

Please select item <1 to 13>

S-Save to FPGA

ETX-204A-AC

*Generator

9. Generator statistics          []

>9  Press "Enter"

ETX-204A-AC

Generator>Generator statistics

>

H-Stop; C-Clear all; F-Clear one; S-Save to FPGA